Appendix 4; Fishing gear plans San Aspiring as specified in CM 22-06; Annex 22-06/A Par 1.2.1

Table 4.  Ranges of line lengths and hook numbers from San Aspiring records averaged over the past three completed fishing seasons in Subarea 88.1 and 88.2.

|  | Minimum | Average | Maximum |
| --- | --- | --- | --- |
| Line length | 1,799 m | 9,922 m | 15,597 m |
| Number of hooks | 1,285 | 7,087 | 11,141 |

Table 5. Description of main gear items

| Item | Description |
| --- | --- |
| Backbone | 12.5 mm lead-core internally weighted polypropylene/nylon line at 50g/m (Fiskevegn AS) |
| Grapnels | 40 or 50 kg - two or four used per line depending on sea, bottom, and tidal conditions |
| Weights | About 5 kg - rarely used, tied to the line occasionally when setting in loose sea ice or when turning while setting |
| Hook type | 15mm straight shank 15/O hooks |
| Chain | Lengths of heavy chain generally 20 or 40 kg used for additional weighting at the line ends. |
| Floats | Only surface floats, either inflatable or pressure floats depending on ice conditions |
| Snoods | 40-50cm blue Capron™ snoods spaced at 1400 mm |