Auto Longlining

The nominated Australian vessel will be targeting Antarctic toothfish (Dissostichus mawsoni) using a 40, 000 hook Mustad Autoline System including a Mustad 3000 Autobaiter baiting machine. The vessel is also fitted with an SP2000 Mustad Autoline hook separator, Autoline Combi haulers, and Mustad Line Controller systems (to reduce the chance of line breakage).

Figure 1: Auto Longline Fishing System

Figure 2: Hook type

Table 1: Antarctic Discovery desciption of main gear items

| Item |  | Descriptions |
| --- | --- | --- |
| Main line |  | 12.0mm Integrated weighted line (IWL) at 50 grams per lead per meter of line |
| Downline |  | 12.0mm line |
| Grapnels |  | 2x80kg grapnels used per line. |
| Hooks | Type | Circle & J-hook hybrid, 2X extra-heavy and extra forged wire, Impregn. Twisted polyester, sewn loop |
|  | Spacing | 1.4m apart |
|  | Length (cm) | 8 |
|  | Shank Length (cm) | 6 |
|  | Gape (cm) | 1.8 |
|  | Throat (cm) | 3.2 |
|  | Front length (cm) | 3.5 |
|  | Spacing (m) | 1.4 |
| Floats |  | Surface floats only |
| Snoods |  | Polyester snoods of 0.45m length |